Post-Dynamic Alignment Test

Locations for averaged GOX Ant. Centers (mm)

|  | Occ. +X |  |  | Occ. -X |  |  | POD +X |  |  | POD -X |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | x | y | z | x | y | z | x | y | z | x | y | z |
| FM1 | 471.39 | -2.45 | 241.08 | -469.47 | -1.97 | 244.47 | 471.71 | -0.71 | -270.13 | -470.49 | -0.38 | -274.20 |
| FM2 | 471.27 | -2.05 | 241.74 | -469.85 | -1.87 | 244.17 | 473.28 | -0.37 | -269.84 | -469.14 | -0.38 | -274.21 |
| FM3 | 471.20 | -2.13 | 241.86 | -469.88 | -2.21 | 243.49 | 472.11 | -0.53 | -269.67 | -470.06 | -0.94 | -274.79 |
| FM4 | 471.42 | -2.37 | 240.99 | -469.25 | -2.11 | 244.71 | 472.00 | -0.42 | -269.93 | -470.51 | -0.69 | -274.20 |
| FM5 | 471.40 | -2.59 | 241.27 | -469.55 | -2.23 | 244.38 | 471.64 | -0.36 | -270.13 | -470.46 | -0.19 | -274.31 |
| FM6 | 471.22 | -1.91 | 241.96 | -469.54 | -2.51 | 244.84 | 471.84 | -1.02 | -269.46 | -470.43 | -0.35 | -274.40 |

Ant. Center: 4pt. average of corners.

# Ant. Normal Angles respect to x-axis (deg.): Tan1(z/x)

|  | Occ. +X | Occ. -X | POD +X | POD -X |
| --- | --- | --- | --- | --- |
| FM1 | 27.41 | -27.34 | -14.84 | 15.16 |
| FM2 | 27.52 | -27.07 | -13.57 | 16.46 |
| FM3 | 27.31 | -27.01 | -14.79 | 15.36 |
| FM4 | 27.55 | -27.15 | -14.97 | 15.00 |
| FM5 | 27.52 | -27.09 | -14.90 | 14.97 |
| FM6 | 27.69 | -27.16 | -14.79 | 14.93 |
| Spec.  Req. | 27.3  1 | 27.3  1 | 15.0  1 | 15.0  1 |

Ant. Normal: the averaged cross-product of 4 vectors pointing from center to corners. It very closes to the measured normal.

The POD’s Ant. center angle respect to x-axis is ~30deg.

Alignment Req. within  1deg. is listed in Req. Matrix 3.2.4.

The followings are the measured three NEA separation points respect to some reference in alignment test

(The SC center is defined by the average of these three points. The measurements in page 1 have been transformed on SC center, where y=0 denoting the SC deck)

| FM1 | X | Y | Z |
| --- | --- | --- | --- |
| COOR-1 | 5.0955 | 3.32 | 19.0165 |
| COOR-2 | -19.0157 | 3.32 | -5.0987 |
| COOR-3 | 13.9263 | 3.32 | -13.9158 |
| FM2 | X | Y | Z |
| COOR-1 | 5.095 | 3.32 | 19.0147 |
| COOR-2 | -19.0126 | 3.32 | -5.1027 |
| COOR-3 | 13.9336 | 3.32 | -13.9059 |
| FM3 | X | Y | Z |
| COOR-1 | 5.0953 | 3.32 | 19.016 |
| COOR-2 | -19.0134 | 3.32 | -5.1047 |
| COOR-3 | 13.9313 | 3.32 | -13.91 |
| FM4 | X | Y | Z |
| COOR-1 | 5.0953 | 3.32 | 19.0158 |
| COOR-2 | -19.0114 | 3.32 | -5.1117 |
| COOR-3 | 13.9314 | 3.32 | -13.9097 |
| FM5 | X | Y | Z |
| COOR-1 | 5.0945 | 3.32 | 19.0128 |
| COOR-2 | -19.0139 | 3.32 | -5.0905 |
| COOR-3 | 13.9184 | 3.32 | -13.9183 |
| FM6 | X | Y | Z |
| COOR-1 | 5.0949 | 3.32 | 19.0145 |
| COOR-2 | -19.0166 | 3.32 | -5.0867 |
| COOR-3 | 13.9276 | 3.32 | -13.9115 |